Four Point Flexure Beam Fixture

by

Nikolay Bobritskiy

Table of Contents

Page

Section 1: Introduction……………………………………………………………6

1.1	Description

1.2	Motivation

1.3	Function Statement

1.4	Requirements

1.5	Success Criteria

1.6	Scope of Effort

1.7	Benchmark

1.8	Success of the Project

Section 2: Design and Analyses………………………………………………….7

2.1	Approach: Proposed Solution

2.2	Benchmark

2.3	Performance Predictions

2.4	Description of Analyses

2.5	Scope of Testing and Evaluation

Section 3: Methods and Construction……………………………………………10

Methods

3.1	Project Solution

Construction

3.2	Description of Construction

3.3	Drawing Tree, Drawing ID’s

3.4	Parts list and labels

3.5	Discussion of assembly, sub-assemblies, parts, drawings

Section 4: Testing Method………………………………………………………13

4.1	Introduction

4.2	Method/Approach

4.3	Test Procedure description

4.4	Deliverables

Section 5: Budget………………………………………………………………..15

5.1	Discuss part suppliers, substantive costs and sequence or buying issues

5.2	Determine labor or outsourcing rates & estimate costs

5.3	Labor

5.4	Estimate total project cost

5.5	Funding source(s)

Section 6: Schedule……………………………………………………………...16

6.1	Tasks, Deliverables, and Total Project Time

Section 7: Project Management………………………………………………….17

7.1 Resources

Section 8: Discussion………………………………………………………….…18

8.1	Design Evolution / Performance Creep

8.2	Project Risk analysis

8.3	Successful

8.4	Project Documentation

8.5	Next phase

Section 9: Conclusion……………………………………………………………20

9.1	Restate your design title and its complete design readiness.

9.2	Restate your important analyses and how this contributes to success.

9.3	Restate your design predicted performance vs actual performance, with respect to your requirements.  Use bullets if appropriate.

Section 10: Acknowledgements……………………………………………….…21

10.1	Gifts, advisors, and contributors

Appendix A: Analysis……………………………………………………………23

a.	Appendix A1: Shear and Moment Diagram for Specimen

b.	Appendix A2: Deflection Increments for Specimen

c.	Appendix A3: Specimen Modulus of Elasticity

d.	Appendix A4: Comparing maximum width and thickness for Deflection and MoE

e.	Appendix A5: Finding Volume of Base

f.	Appendix A6: Base Volume 2

g.	Appendix A7: Mass of Base

h.	Appendix A8: Volume of Bend Points

i.	Appendix A9: Mass of Bend Points

j.	Appendix A10: Volume of Instron to base connector

k.	Appendix A11: Mass of Connector

l.	Appendix A12: Approximating Total Cost

Appendix B: Drawings…………………………………………………………..35

a. 	Appendix B1: Bend Point Part

b.	Appendix B2: Instron to jig attachment

c.	Appendix B3: Bottom Frame

d.	Appendix B4: Top Frame

e.	Appendix B5: Bottom Assembly

f.	Appendix B6: Top Assembly

g.	Appendix B7: Top and Bottom Assembly

Appendix C: Parts List…………………………………………………………....42

Appendix D: Budget……………………………………………………………...42

Appendix E:	 Schedule……………………………………………………………43

Gannt Chart

Appendix F: Expertise and Resources……………………………………………45

Appendix G: Testing Report……….……………………………………………..46

Appendix H: Resume/Vita………………………………………………………..54

Appendix J: Safety………………………………………………………………..55

Section 11: References……………………………………………………………57

Abstract

Structural design is one of many important aspects in mechanical engineering. Newly discovered composites are making their way into the engineering world. These materials have certain properties which need to be tested in several ways before they can be put to application. One of such tests include the four-point bend test. The fixtures that can be purchased currently can be expensive, typically ranging from $700-$1,000. The University has been struggling to afford commercial jig at this cost and would be helpful if one were available. The device consists of a 6061-aluminum base making it light, cheap, and faster to manufacture than other alternatives. The base secures to the Instron with pins, making it a quick process. The 4 contact points are made from A-36 steel which can be easily assembled to the base with a socket head screw. The device was thoroughly analyzed beforehand to withstand a maximum load of 1000 lbf, which easily met that requirement. Upon project completion the device total cost totaled to $246, or much cheaper than what is available for purchase. Testing will consist of assembling the fixture and installing it onto the Instron. As well as performing a four-point bend test on a known material such as aluminum to see the difference in percent error between actual bending stress and experimental.

1. Introduction

1.1 Description

The four-point bend fixture is a fixture that is made to test material flexural strength. It is important in many engineering scenarios to understand behavior of materials under certain conditions so that they are built for certain applications. The 4-point bend test is made to bend the material and measure stress at the point of failure by measuring force vs deflection on an x-y graph.

1.2 Motivation

This project was motivated by providing a 4-point flexure beam to Central Washington University as the University needs one for their universal testing machine. The motivation also comes from learning about 4-point bend interest in observing what occurs to different materials when they are bent.

1.3 Function Statement

The device is needed to perform a 4-point bend test to test material bending strength and to determine the modulus of elasticity for a material.

1.4 Requirements

The requirements and design of the project will follow ASTM standard for a 4-point jig. Along with the following requirements

- Each component weight under 10 pounds

- Part assembly less than 20 pounds

- Cost under $600

- Size constraint within the Instron

- Withstand 1000 pounds of force

1.5 Success Criteria

Perform the bend test and compare measured and calculated bending modulus to known value of a specimen.

1.6 Scope of the Effort

Will include bottom 2-point fixture, upper 2-point fixture (4-point bend), and upper single point support (3-point bend).

1.7 Benchmark

The three- and four-point bend fixtures already exist the problem is that they are very expensive, So the benchmark would be to minimize the costs of the project. Most 3-4-point bend jigs can be as much or more than $700, the goal is to keep it under $600.

1.8 Success of the Project

Success depends on the capability of measuring the flexural stress on universal tensile machine and comparing the tested values.

2. Design and Analysis

2.1 Approach

The idea of the design was planned by examining ASTM standards of a 4-point jig in E855 (page). Initial idea of the design was also suggested by Dr. Johnson. To accommodate the requirements, the assembled part of the bottom section of the 4-point bend must weigh under 20 pounds. In this case the base of the jig was determined would be best to be made of a light material such as Aluminum. While the contact points of with the specimen will be made out of a stronger material such as Steel. The jig will be able to perform the test on specimens measuring

2 in x 12 in. The 4-point jig will look something like this:

2.2 Benchmark

The benchmark for this project is to compare existing 4-point jigs on the market and reduce the cost by around $100 dollars. The most important requirement is the jig must mount on to the Instron machine. Therefore, precision is very important when designing the mount of the jig.

2.3 Performance Predictions

The 4-point bend jig will secure onto the Instron. The bottom section of the jig will be the heaviest and will have to meet the requirement of weighing under 20 lb., so it shouldn’t be a problem to install.

2.4 Description of Analysis

Appendix A1:

Drawing Free Body Diagram of all forces acting on a specimen and finding shear and moment diagrams of the specimen under the maximum load of 1000 pounds. This is repeated to find the maximum moment possible, which would occur when the length between the top two supports is at a minimum of 2 inches apart or 1 inch from the midpoint.

Vmax = 250 lb

Mmax = 1250 lb*in

Appendix A2:

Finding Permanent Deflection for four-point loading

Permanent Deflection = 0.276 in

Appendix A3:

Finding shear and moment diagram for the base

Vmax = 250lb

Mmax = 1500 lb

Appendix A4:

Finding normal stress, deflection, shear stress, and bending stress

Normal Stress = 83.33 psi

Deflection =0.000117 in

Bending Stress = 750 psi

Shear Stress = 125 psi

Appendix A5–A6:

Finding the volume of the base. Threads are simplified into cylinders to simplify finding the volume. The base is divided into two sections to simplify the calculations.

Volume =94.02 in^3

Appendix A7:

Finding the total mass of the part to determine if the part fits the requirement of weighing under 10 pounds. The base will be made of 6061 aluminum and 0.0975 lb/in^3 density is used. Also, the total cost of the raw stock material for the base of the top and bottom assembly is found by looking at the prices on Midweststeelsuppy.com.

mb = 9.167 lb

Base Cost Bottom = $73.63

Base Cost Top = $65.03

Appendix A8-A9: Finding the volume of the supports. There is four supports total, two on each assembled part. Finding the Mass and the cost of the supports.

Vsupport =3.47 in3

Msupport = 0.97 lb

Cost = $14.67

Appendix A10: Finding Volume of the bottom attachment and approximating the volume for top attachment:

Vbottom attachment = 4.01 in^3

Vtop Attachment = 4.01 in^3

Appendix A11: Finding Mass and Cost of the Attachments

Mbottom attachment = 1.288 in3

Mtop attachment = 1.288 in3

Costtop attachment = $8.85

Costbottom attachment = $8.85

Appendix A12: Determining and approximating total cost and finding the mass for the bottom assembly and approximating total mass

Cost Total = $255.04

Mass Bottom Fixture = 12.31 lb

Mass Top Fixture = 11.5 lb

Total Mass = 23.81 lb

3. Method and Construction

Methods

The engineering discipline areas of interest comes from courses such as machining, mechanics of materials, mechanical design and material science. First and foremost, the initial design of the project is constrained by requirements such as cost, weight and performance. The initial design was conceived with the idea that the jig should be light enough to be easily installed onto the Instron. The initial requirement that the project must weigh under 20 pounds and per assembly as well as cost less than other readily available 4-point bend jigs was easily met through engineering analysis of volume, mass and cost (Appendix A5-A11). The analysis on the requirement that the jig must withstand 1000 pounds of force was performed in Appendix A3-A4. It was discovered that the normal stress acting on the base would only be about 83.33 psi which is much smaller than aluminum yield stress of 35,000 psi. Similarly shear stress on the base was 750 psi, which is much smaller than maximum shear of 30,000 psi. The project will further be optimized to reduce the weight, the initial idea is that the base of the jig can further be reduced in height to reduce the weight. More calculations of stress and deflection will need to be performed but realistically the ASTM standard specimens are so thin (0.05 inches maximum height) that reducing base by perhaps as much as two times the initial design wouldn’t cause anywhere near the maximum yield and shear stress on the aluminum base.

3.1 Project Solution

The 4-point bend jig project was planned, analyzed, and designed at Central Washington University. The project will be constrained to CWU resources and closely follow the requirements. The parts will be purchased in form of raw stock material and machined following ASTM standards and requirements.

Construction

3.2 Device Construction

The 4-point bend jig will be assembled from machined parts performed at CWU. The entire project will consist of 2 assemblies which is the bottom portion of the 4-point jig and the top portion of the 4-point jig as well as 1 part for the top portion of a 3-point bend jig. The parts will be obtained from suppliers in form of raw metal and machined to ASTM standards. The first assembly (Figure 5: 4-Point Bottom Jig) will include the stock body which will secure two 30˚-supports to the stock body (secured with screws) as well as a bottom portion which connects to the stock body and the Instron. The second assembly will follow the same procedure as the first assembly, but the stock body will be smaller in length.

3.3 Drawing Tree

(Figure 1 Decision Tree)

3.4 Parts List

| 4-point bend base Bottom | The base of the entire jig supports the bottom assembly. Appendix B3 |
| --- | --- |
| 4-point bend base Top | The Base of the entire jig supports the top assembly Appendix B4 |
| Supports x 4 | 30˚ Supports with a 0.005 radius. This will bend the material. Appendix B1 |
| Hex Screw x 4 | Hex Screws that will attach the supports to the base |
| Pins x 2 | Will be used if design 1 is used. Will attach base to the Attachment point which will secure the bottom jig. |
| Instron to Base Attachment Top | Will secure the top base to the Instron. Appendix B6 |
| Instron to Base Attachment Bottom | Will secure the bottom base to the Instron by using pins if initial design is used. Appendix B2. |

3.5 Parts and Assembly

The 4-point bend jig will consist of 2 assemblies, which will be very similar. The bottom assembly consist of a base (Appendix B3), this acts as the frame of the jig. The base will attach to the piece which secures the base to the Instron (Appendix B2). The primary design is that the base and the attachment will be secured with a pin, or the secondary idea for the design is instead to just weld them together, and have it become a subassembly. The base will also hold two supports (Appendix B1). The supports will attach to the base with Socket Hex Screws which will be purchased from McMaster Carr. These will be partial threaded ideally ¼-28 with a total length of 2 inches. The complete assembly of the bottom jig can be viewed in Appendix B5. The Top assembly will follow the same assembly, but the size of the base and attachment part will be slightly different to accommodate dimension requirements of the Instron.

Design Issues

Part 1

Some design issues that came up during manufacturing process was modifying the design because the raw material was a bit larger than the initial design, so in order to save time and not have to mill 0.5 inches from a 3.5-inch x 14.5 inches. It would be simpler to keep this size and finish the project first and the perhaps later mill of excessive material if there is time. Another problem that came up during milling, reducing the length of the steel from 2.5 inches to 2.18 inches on a (2 x 1.25) inch piece took more time than anticipated. The prediction was that it would take 4 hours, but because of multiple cuts and reducing size by 0.01 inches for each cut took way longer than expected, about 8 hours. Otherwise the project is moving along and getting close to finishing 4 parts.

Part 2

Design changes were made for the project in the past 4 weeks. These design changes include the following: reducing the height of the groove on top base and bottom base from 1 inch to 0.5 inches. Modifying the diameter of the slot hole where the attachment point is inserted from 1.25 inches to 1 inch – the reason for this change is because only one-inch end mills are available, to make the hole larger a different method would need to be used. This also means that the attachment point will now need to be turned to about 1 inch instead of 1.25 inches. The last design modification that occurred because of an issue. Since the A36 steel (Point Bends) material was hot rolled, it caused a problem when milling a 30-degree angle using a 30-degree angle tool. This made some angles turn out a bit less or more than 30 degrees, but it shouldn’t be a problem when the fillet is created. The only problem is that the height of the parts at the longest section was different, so they had to be milled to the same size of 1.9 inches, which took about an extra 2 hours.

4. Testing Methods

4.1-4.3 Introduction, Approach, and Testing

The 4-point jig will be tested. Testing will be done on Instron at Central Washington University on an Instron machine. The test that will be performed is installing the jig onto the Instron and performing a bend test on a measured specimen of a known material such as steel or aluminum. Using the found values of flexural stress and flexural strain to determine the flexural modulus of elasticity of the specimen and comparing the to the known book value. The testing will follow the procedure outline in E855.

The 4-point fixture will also need to be weighed on a scale to make sure it does not exceed the maximum weight of 20 pounds for both assemblies.

Testing

The testing that will be performed includes the following. First weigh in all the pieces to check if it meets requirement for each part to weigh under 10 pounds. Check if every hole in top and bottom base can be properly secured with the bending points. There are 12 holes total that need to be checked. A bending point can be secured with a ¼-28 head cap screw and checked if the bending point sits flush. This also means that the socket head screw head should also sit flush in order to secure the point bend to the base. Next securing the attachment that holds the base to the intron will be checked. The attachment attaches to the base with a pin and must fit and sit flush for this to work. Same will be done with the Instron and the assembled part. Next the assembled part will be checked to see if it meets the weight requirement of less than 20 pounds per assembly. Lastly check if the both assemblies properly secure to the Instron and sit flush.

Testing Update 1

One of the requirements was to make sure that the jig assembly fits within the Instron in Hogue Hall 127. The test was performed by assembling the parts and attaching both the top and bottom assemblies onto the Instron with the pins. Both parts that fit onto the Instron were manufactured to the same length of 1.77 inches (Appendix 2B). Although the bottom piece fit in perfectly onto the Instron the top piece did not. The reason for this is that the setup with the Instron itself has two different bolts that attach onto the top and bottom. The one on the top has slightly larger length on the head of the screw. This means that when the jig is inserted the distance with the pin is slightly misaligned. This is fixable in two ways. One would be to face of both jig insertions. Since on the bottom it sits on its middle portion where on top the top face pushes off on the bolt. The other way would be to just face one of the insertions and label as top and bottom attachment. Another the requirements were that every component must weigh under 10 pounds, and that each assembly should weigh under 20 pounds. The first test that was performed was to check the weight of each component. All eight of the main components that was checked for mass includes the following and their respective mass: bending point 1: 1.2 lb   , bending point 2: 1.2 lb,    bending point: 3: 1.2 lb , bending point 4:  1.2 lb , base top: 10.1 lb   , base bottom: 11 lb   , jig attachment bottom: 0.4 lb, jig attachment top: 0.4 lb,. Next the top assembly was assembled with 2 bending points, top base, jig attachment top, 2 pins (ø-0.5 in x 3.5 in), ¼-28 head cap screws length 2.25 in. The mass was then of the top assembly was then checked and came out with 12.9 lb. Similarly, the bottom assembly was assembled in the same way and the mass was measured to be: 13.8 lb. The results can also be seen under appendix C.

Testing Update 2

During the second part of the testing phase for the project, some of the issues that occurred during testing includes testing for 1000 lbf requirement. As Central Washington University is currently closed, and the equipment needed for the test includes the Instron machine located in Hogue Hall. Previously TA Jim Helsius assembled the fixture onto the Instron for the testing the requirement of size constraint within the Instron. Both assemblies assemble onto the machine now. Another problem that occurred was to create video and test for the mass requirement the fixture had to be recovered and further testing had to occur outside of University.

5. Cost and Budget

5.1 Suppliers, Cost, and Issues

The project will be managed by first ordering the parts from sources such as Midweststeelsupply.com. These parts will come as a raw stock material in a form of plates and bars. The raw stock material will then be machined to the designed parts in drawings in Appendix B. Some of the parts such as hex screws or pins will be ordered from Mcmaster.com. These parts will be used to assemble the machined parts to create the 4-point jig. The list of all the parts and their cost can be seen in appendix D.

The most important part when creating the machined parts is that dimensions must be accurate and so there is very little to no room for error. A single mistake can make the part useless which means new stock will have to be ordered and that results in lost time and increases the cost of the project.

Actual Budget

As of January 7th, 2020, all raw material was purchased. The expected cost of raw material was $214.5 plus shipping cost. The actual cost of the raw material was $211.27 with shipping included in this cost. The reason why this price was lower than expected was because when ordering raw stock in bulk it costs less then the prices shown for individual item. The project requirement is that it must be less then $600. So far only 2 pins are left to purchase which is approximated to be around $10 plus shipping. Due to change of the design due to advising from Matt Burvee, instead of buying hex screws, the design was changed to socket cap screws (1/4-28). The approximated cost for these screws is about $30 plus shipping.

So far the expected cost of the project is $250-$300. Which is way below the requirement of under $600.

Budget Cost Issues

For the budget there was not much issue when it came down to meeting the cost requirement for the project. Majority of the research was done before ordering the parts and a rough estimate was made for the cost. It was previously predicted that the cost of the project was going to be between $250-$300 dollars. With an estimated guess of $251.27, the project came out to be at $241.36 which is even cheaper than the estimate. The reason for this is because some ordered parts were identical, and when ordering in ‘bulk’ from Mid-West Steel Supply meant that there were discounts on those parts, and they came out cheaper then initially expected.

6. Schedule

6.1 Tasks, Deliverables, And Total Project Time

The first task is to order raw stock for the project early, so that they are available for winter quarter. The stock will be ordered in early December. Next five parts must be machined by the end of week three in winter quarter. The project consists of eight parts totals excluding the socket hex screws and pins. This means that majority of the project shall be completed very early, this will help reduce the load later in the quarter or if some issues arise this can be resolved earlier rather than later. By week five 70% of the project must be completed which should not be a problem if 5/8 parts are made in week 3. This means that this means that only 1 part needs to be made to reach that requirement. By week 7, the entire project shall be completed and assembled.

To fulfil the deadline requirement for the project listed above the parts that shall be completed by end of following weeks is listed in Appendix E.

The total time to complete the project is estimated to be 160.5 hours. This estimation is done from the Gantt chart in Appendix E for quarter 1 which took 53.5 hours to complete.

Scheduling Issues

Scheduling issues that occurred during manufacturing phase of the project include the following: estimating times for manufacturing parts vs making the parts can be completely different. For example, when making the first four parts it was estimated that it would take about 2 hours to make each part, when in fact it took nearly 7 hours per part. The main reason for this is because of changes made as well as reducing the length of the part from 2.5 to 2.18 inches with several passes and only .01 inches taken of per pass. This results in the part taking much longer to make than expected. Other scheduling issues that arose, include expecting to start making one part, but then due to complications a different part had to be made first. For example, the initial design of the base had a hole designed to 1.27 inches in diameter so that a 1.25 base attachment could fit inside. This had to be changed because making a hole of that size requires learning new techniques, so instead the hole was changed to 1.00 inch, this means that the base attachment now had to be turned by about .26-.27 inches more to fit inside, this takes more time then the initial design.

During the spring quarter, some scheduling issues that occurred for the project are mostly related to COVID19, as the University was closed during this time. One of the issues with this is performing the necessary tests for the project, which includes testing the requirement for the 1000 lbf. In order to perform this test, the Instron machine is required which is located at CWU. To perform other tests and take videos and pictures of the project, the project had to be recovered from the University by contacting Professor Pringle and allotting the time to make the drive from western WA.

7. Project Management

7.1 Resources

The most important resource is the person working on this project, so safety is number one priority. Safety of others is just as important as the safety of the engineer that is why it is important to be aware of dangers and surroundings so they can be avoided. Other resources include the following mentors: Doctor Johnson, Matt Burvee, Professor Pringle, Professor Choi, Jim Helsius, Ted Bramble and the Central Washington University resources of staff and equipment. This equipment includes computer labs and classrooms where the project is designed, analyzed, modified, and built. Other important resources include SolidWorks, Microsoft Word, Microsoft Excel, and Machining lab that are made available by CWU. The 4-point bend fixture project will be funded by engineering student. With all the available resources available, the project will be successful.

8. Discussion

8.1 Design Evolution

The proposal for the project was introduced during week 1 of Fall Quarter. The initial design started out as depicted under Design and Analysis. The initial design was going to be made entirely out of steel, but Dr. Johnson suggested that the base could be made from aluminum to make it lighter. During week four of quarter one the design was modified again (Appendix B5). The base of the project was modified with constrained walls on the sides. This was done with the idea that the when assembling the supports to the base they would be kept perfectly aligned at a 90-degree angle. Another thing that was modified was the supports, or more specifically how they attach to the base, it was decided that it would be best to secure them with a socket hex screw. The supports were also modified to fit the ASTM standards, the change that was made was to put 0.005-inch radius fillet on top which will bend the material. The attachment was also modified to be made out aluminum, so the only part that will be steel is the supports that will bend the material.

8.2 Project Risk Analysis

The two most important risks to consider is project management and schedule. The project is constrained to be completed in 10 weeks during winter quarter, this means that there is very little room for error when machining the parts. Mistakes will not only waste a lot of time but also increase the costs substantially. Some of the risks will include, making proper analysis, taking correct measurement so that the fixture can be assembled and attach to the Instron, Machining correct parts, taking relevant data during testing, and doing correct research to extend the knowledge and expertise on the project. The project will be successful if the outlined schedule in Appendix E is followed. As well as understanding risk analysis and planning with proper task to take during designing and machining of parts. Also, following the safety outlined in Appendix J will also mean the project can be done safely.

Manufacturing Issues/Modifications

There were several issues that occurred for the manufacturing phase of the project in the first half of the project. The first big issue that occurred was that the design had to be slightly altered become the raw stock material is slightly larger than that for which was designed. The 4 points that bend the material were designed for a width of 1.00 inch, but the stock material was 1.25 inches, because of this it was discussed with advisor Matt Burvee that it would be better to just keep the material at 1.25 inches to save time. The design for the length of the material was 2.18 inches, this had to be machined down from a length of 2.5 inches for raw stock. The issue that occurred here is that because the material is A36 steel it took quite a bit of time to do this. The main reason is because on the mill only 0.01 inch could be removed at a time, as well as 2 passes had to be made for each length reduction. It was predicted that this would take 4 hours, but it took more like 8 hours. Similarly making the 30-degree angle cut had the same issue, instead of predicted 2 hours, it took more like 4 hours. Since it took more time than expected the project fell a bit behind schedule. For the following week it is expected to bring in 50% of the finished parts. This may not be a problem since there is only 8 total parts, and four are nearly done.

During the second half of manufacturing the project some of the issues that occurred was mostly human error. For one of the cylindrical parts, the diameter has a very tight tolerance of 1.24 in +/- 0.003 in or so. Initially the part was turned to a diameter of 1.25 in, but it couldn’t fit into the Instron. So about .01 in had to be taken of to be within tolerance. In the process of doing so something must have gone wrong when touching of the zero from the side of the cylinder because instead of the material being 1.25 in it went all the way down to 1.215 in. This must have happened because of some human error when setting up the part. So, what happens when this diameter cylinder is inserted into the Instron without a tight fit, it wobbles around side to side. In order to fix this problem, the first solution was to use knurling. This only added about .005 inches to the diameter although it helped it was still nowhere near the desired diameter. The next step that was taken to resolve this issue was to take an electrical tape and wrap it around to increase the diameter. This increased the diameter to about 1.238 which is in desired range. Although it is not perfect it works.

Aspects of Testing

The project was a success. It meets the first requirement of the ability to be installed onto the Instron. Although some dimension tolerances could be improved, such as reducing the height of the cylinder that attaches to the Instron, as this will allow the pin to slide in easier (Appendix B). The total cost of the project came to be $241.36 which easily met the requirement of $600 dollars. Most four-point flexure fixtures are around $700-$1000 dollars. In order to easily install the fixture each assembly had to be under 20 pounds. This requirement was also met with the bottom assembly having a mass of 13.8 lbm and the top assembly of 12.9 lbm.

Conclusion

The 4-point bend fixture will be a successful project because the engineer has the resources readily available through Central Washington University. Mentors including Doctor Johnson, Professor Pringle and Professor Choi will guide him along the path to success. As well as the expertise developed throughout the time of coursework to make this project possible. The fixture will meet all the requirements through design and engineering analysis. The engineering analysis performed on the fixture contributes to meeting the requirements of cost, mass, dimensions, and structural integrity for the project to be successful. The 4-point bend fixture will fit onto the Instron and be ready to take measurements of 6 aluminum specimens.

10. Acknowledgments

Advisors

Matt Burvee

Dr. Johnson

Professor Pringle

Professor Choi

Jim Helsius

Ted Bramble

Huy Dinh

Appendix A: Analysis

(Appendix A1: Shear and Moment diagram example)

(Appendix 2A: Permanent Deflection)

(Appendix 3A: Base Shear and Moment Diagrams)

(Appendix 4A: Fining Normal, Bending Shear Stress, and Deflection)

(Appendix 5A: Determining Volume for Aluminum Base)

(Appendix 6A:  Base Volume Continued)

(Appendix 7A: Finding Base Mass)

(Appendix 8A: Support Volume)

(Appendix 9A: Support Mass and Cost

(Appendix 10A: Attachment Volume)

(Appendix 11A: Mass and Cost of Attachments)

(Appendix 12A: Total Cost of Materials)

Appendix B: Drawings

(Appendix B1: .005 in point bend)

(Appendix B2: Instron to jig attachment)

(Appendix B3: Bottom Frame)

(Appendix B4: Top Frame)

(Appendix B5: Bottom Assembly)

(Appendix B6: Top Assembly)

(Appendix B7: Top and Bottom Assembly)

Appendix C and D: Parts List and Budget

Parts List and Budget

Appendix E: Schedule

Task Dates

- Week 1: Machine Base Bottom

- Week 2: Machine 2 Supports

- Week 3: Machine 2 Supports

- Week 4: Begin Machining Top Base

- Week 5: Finish Machine Bottom Base, Begin Machining Attachment Bottom

- Week 6: Finish Attachment Bottom, Start Machining Attachment Top

- Week 7: Finish Machine Attachment Top, Assemble Project

- Week 8: Machine Single Support for 3-Point Bend

(Appendix E1 Tasks Dates)

(Appendix E2: Gantt Chart)

Appendix F: Expertise and Resources

Senior project was designed, analyzed, manufactured, and tested at Central Washington University during 2020 as a Senior Student.

Appendix G: Test Report

Introduction

The requirements and design of the project will follow ASTM standard for a 4-point jig. Along with the following requirements

- Each component weight under 10 pounds

- Part assembly less than 20 pounds

The parameter of interest for mass of the parts and assembly is to maintain a light jig for a person to install onto the Instron without much effort. The predicted value is that each assembly is around 12-pound mass and heaviest part around 10-pound mass. The data will be collected using a mass scale.

- Cost under $600

The total cost of the project is $241.36 which is well under $600 requirement.

- Size constraint within the Instron

The parameter of interest for size constraint is that the assembly can be assembled and fit onto the Instron in a timely fashion of 5 minutes. The predicted value is that both assemblies can be assembled and fit onto the Instron in around 3 minutes. The data will be collected by having both assemblies unassembled and timed with a stopwatch for how long it takes to assemble both fixtures.

- Withstand 1000 pounds of force

The parameter of interest is to check if the jig can withstand 1000 pound of force (maximum Instron force) without any damage. The aluminum jig is most likely to fail at the attachment part of the Instron where it is held just above the pin. The predicted value is around 31000 pounds of force from stress/area calculation.

Method/Approach

The testing predictions is that the jig is easy to lift and install for a single person, the entire assembly assembles without any issues, and can withstand at least 1000 pounds of force to perform the four-point bend test. The calculated parameters are mass, time to assemble the jig, and withstand a load of 1000 pounds of force. The mass scale will have an accuracy of +/- 0.1 pounds. The time will be measured with human error with a +/- 1 second. To measure if the jig can withstand 1000 pounds of force an object will need to be placed in between that can withstand 1000 pounds and tested on the Instron to +/- 1 lbf. The tools/resources that will be needed to perform the requirement test are a mass scale, stopwatch, Instron, L-key 3/16, 4 pins, and two jig assemblies consisting of base, 2 contact points, and cylindrical attachment. The data will be recorded with videos or pictures. The numerical data will be recorded into data tables where it will be stored and analyzed. Some operational limitations such as using the Instron might not be available to perform as the University is closed for spring quarter.

Test Procedure

Summary

- Duration for Setup: The entire setup can take between 5-10 minutes, Time to complete test an additional 2-3 minutes.

- Place: Hogue 120

- Resources: Instron Machine, Hex L-key 3/16, 4x (¼-28 thread, 2-1/4 length socket head screw). Mass Scale.

- Risk, Safety: The entire assembly is around 20-30 lbs. That is why one should be careful when attaching it to the Instron. Securing all the parts properly is a must. Otherwise the device could fall and cause injury or damage the machine.

- Discussion: During the Initial setup it turned out that the bottom base of the Instron and top base of the Instron where the assembly is attached has slight variation in height (Figure 3). This is because the bolt head height is different which attaches the on the Instron itself. When inserting the cylindrical part (Figure 2) into the bottom base (Figure 3) of the Instron and putting in the pin produces good results, it sits perfectly, but taking the same cylindrical piece an inserting it onto the top section of the Instron base causes it to stick out more, so the pin cannot be inserted. This means that either one of the two-cylinder heights must be faced slightly more, or both must be faced slightly.

- Setup

Note: Figure 4 provides how the entire assembly should look like attached to the Instron. The shorter base goes on top, and the longer base on the bottom.

- Use 3/16 L-hex key to and ¼-28 thread, 2-1/4 length socket head screw to attach bending point to the base in desired location. The longest length of the bending point facing inward (Figure 1).

L-Hex Key

Point Bend

Socket Head

Base

Figure 1: Base and Bending points assembly

- Attach the cylindrical part with the largest diameter side onto the Instron with a short pin. (Figure 2 and 3)

Cylindrical Part

Figure 2: Cylindrical Part

Instron Base

Figure 3: Instron Base Bottom

- Attach the base onto the cylindrical part that was inserted in step 2 using a 0.5 in diameter pin with a length of 3-1/4 in Should look similar to Figure 4 but with pins and inside the Instron.

Figure 4: Base attached to the cylindrical part.

- Perform the 4-point bend test at 1000 pounds force load limit

- Disassemble

- Weigh individual parts on the mass scale

Deliverables

In conclusion the project can be assembled quickly and single handedly, it is easy to assemble and install onto the Instron. The top base and bottom base did not meet the requirement of being under 10 lb, but the entire assembly did meet the requirement of 20 lb. The time to assemble both fixtures took 3.2 minutes. Testing for 1000lbf cannot be completed as CWU is closed.

Report Appendix

Appendix G1: Procedure Checklist

- L-hex key 3/16

- 4 pins ø-0.5 in, 3.5 in length

- 4 hex cap screws ¼-28, 2.25 in length

- Stopwatch

- Mass Scale in lb

- Instron

Appendix G2: Data Sheet

Data Form

| Part | Mass |
| --- | --- |
| Top Assembly |  |
| Bottom Assembly |  |
| Top Base |  |
| Bottom Base |  |
| Bending Points |  |
| Cylinders |  |

| Assembly Time |  |
| --- | --- |
| Withstand 1000 lbf (Yes/No) |  |

Appendix G3: Raw Data

Parameter Values:

| Part | Mass |
| --- | --- |
| Top Assembly | 13.8 lb |
| Bottom Assembly | 12.9 lb |
| Top Base | 11.0 lb |
| Bottom Base | 10.1 lb |
| Bending Points | 1.2 lb each |
| Cylinders | 0.4 lb each |

| Assembly Time | 3.2 minutes |
| --- | --- |
| Withstand 1000 lbf (Yes/No) | - |

Calculated Values:

| Part | Mass |
| --- | --- |
| Top Assembly | 12.31 lb |
| Bottom Assembly | 12.31 lb |
| Top Base | 9.17 lb |
| Bottom Base | 9.17 lb |
| Bending Points | 0.97 lb each |
| Cylinders | 1.12 lb each |

| Assembly Time | 3 minutes |
| --- | --- |
| Withstand 1000 lbf (Yes/No) | - |

Success Criteria Values:

| Top Assembly | 20 lb | Success |
| --- | --- | --- |
| Bottom Assembly | 20 lb | Success |
| Top Base | 10 lb | Fail |
| Bottom Base | 10 lb | Fail |
| Bending Points | 10 lb | Success |
| Cylinders | 10 lb | Success |

| Assembly Time | 5 minutes | Success |
| --- | --- | --- |
| Withstand 1000 lbf (Yes/No) | - | - |

Appendix H: Resume

Nikolay Bobritskiy

2214 119th Ave SE, Lake Stevens, Washington 98258

Cell: 425-322-9828

Email: Nikolay_5@yahoo.com

OBJECTIVE:  Pursuing Bachelor of Science in Mechanical Engineering and Technology and working towards mechanical engineering 1 position.

EDUCATION:

Everett Community College, Everett, WA

Associate of Science in Engineering (Graduated 2018)

Central Washington University, Ellensburg, WA

Bachelor of Science – MET (Expected Graduation June 2020)

PROJECTS:

Balsa Bridge Wood Project

- Built Balsa Wooden Bridge for class competition

- Explored Strongest bridge structures

Electronic Robot

- Built Robot using Arduino board and MATLAB for self-driving robot to place medicine boxes in rooms specific rooms

- Won 1st place for most effective robot.

Electric Motor

- Made small electric motor and measured its efficiency for class project/presentation.

WORK EXPERIENCE

Sharp Electric - Electric Apprentice: Everett WA (Summer 2016)

VOLUNTEER EXPERIENCE

- Helped clean high school campus

- Volunteered at Snohomish Library

- Helped teach Tae Kwon Do Class

- During elections helped Representative Hans Dunshee

  - Make posters for school campus

  - Hand out flyers

COMPUTER SKILLS

- Proficient with Microsoft Word, Power Point, Excel

- Experience with SOLIDWORKS and AutoCAD

Appendix J: Safety

Safety Procedure

The safety procedure of operating and performing a 4-point bend test requires the user to wear safety goggles, and perhaps steel toed boots. Although a 4-point bend test is not very dangerous to perform, safety goggles should still be used as a precaution in the lab. The steel toed shoes should be used in case an operator drops the part or something else while installing it onto the Instron.

Designing the part Safety

For design of the 4-point bend same rules apply to the safety procedure. One must always wear goggles and steel toed shoes. As well as hearing protection if operating loud machinery. Majority of the project will require to operate the mill, CNC, and drills which can all release metallic fragments that can be dangerous to the eye. Steel toed shoes are required in case the operator drops the material.

JOB HAZARD ANALYSIS

| Prepared by: Nikolay Bobritskiy | Reviewed by: Approved by: |
| --- | --- |

| Location of Task: | Machine Shop |
| --- | --- |
| Required Equipment / Training for Task: | Safety Goggles, Appropriate Footwear, Hearing Protection |
| Reference Materials as appropriate: |  |

| Personal Protective Equipment (PPE) Required (Check the box for required PPE and list any additional/specific PPE to be used in “Controls” section) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Gloves | Dust Mask | Eye Protection | Welding Mask | Appropriate Footwear | Hearing Protection | Protective Clothing |
| Use of any respiratory protective device beyond a filtering facepiece respirator (dust mask) is voluntary by the user. |  |  |  |  |  |  |

| PICTURES (if applicable) | TASK DESCRIPTION | HAZARDS | CONTROLS |
| --- | --- | --- | --- |
|  | Drilling | Eye injury from metal debris Injury caused by breaking the bit | Wear eye protection. Do not use compressed air. Feed with the appropriate pressure. Use the appropriate bit for the type of metal. Wear eye protection. |
|  | Milling | Possible eye injury from wire stitches thrown out by milling blade | Wear safety glasses during operation |
|  | Turning | Injury to hands Possible eye injury from wire stitches thrown out | Never disconnect safety Shields Wear safety glasses during operation. |
|  | Facing | Injury to hands Possible eye injury from wire stitches thrown out | Never disconnect safety Shields Wear safety glasses during operation. |

(Figure 12: Safety Chart)

References

ASTM 3-4 Point Bend Standards (E855) Reference Material

(page 792)

(Page 793)

(Page: 794)

(Page 795)

(Page 796)